SIH 2026 – SIH26001

AI-Based Early Warning and Landslide Risk Monitoring System in NER

Complete Project Blueprint: Problem → Data → AI/ML → GIS → Alerts → Field Response

| Category | Software |
| --- | --- |
| Theme | Disaster Management |
| Organization | Ministry of Development of North Eastern Region (MDoNER) |
| Region | North Eastern Region (NER), India |
| Project type | AI + Geospatial Intelligence + Real-Time Web/Mobile Platform |

This is an implementation blueprint based on SIH26001 and public data documentation; it is not an official MDoNER/SIH solution.

# 1. Problem Statement – Simple Explanation

The North Eastern Region frequently faces landslides, flash floods, road blockages and slope failures. Heavy rainfall, fragile terrain and unplanned hill cutting can destabilize slopes. These events can damage roads and infrastructure, isolate villages and delay emergency response.

The core problem is that monitoring is often reactive: authorities may learn about a dangerous situation after it has already happened. SIH26001 asks for an AI-enabled platform that combines environmental and historical information to identify vulnerable areas, predict possible landslide events, visualize risk, issue warnings and collect field/citizen reports.

## What the SIH problem expects

Collect rainfall, soil-moisture information, satellite imagery, terrain/slope data and historical landslide records.

Use AI/ML to identify high-risk zones and predict possible landslide events.

Provide real-time alerts to district administrations, disaster-management authorities and communities.

Use GIS to visualize vulnerable roads, villages and infrastructure.

Allow geo-tagged photo/video reports of cracks, slope movement and blocked roads.

Show risk severity, road connectivity, weather-linked risk and emergency prioritization.

Support multilingual notifications and low-network/offline operation.

# 2. Proposed Solution – Landslide Intelligence & Early Warning System

The proposed platform creates a dynamic risk score for each geographic zone by combining static terrain susceptibility with current environmental conditions and field evidence.

Historical landslides + Terrain + Satellite + Rainfall + Soil moisture + Field reports → AI model → GIS risk map → Alerts → Response

# 3. High-Level Architecture

DATA SOURCES
 ├─ Sentinel-1 / Sentinel-2 / Landsat / ISRO-Bhuvan
 ├─ SRTM / Indian DEM
 ├─ NASA GPM IMERG + available official weather feeds
 ├─ NASA COOLR / Global Landslide Catalog + regional inventories
 ├─ Soil-moisture products / optional sensors
 └─ Our mobile/web field reports

          ↓ Data ingestion & preprocessing
          ↓ Feature extraction
          ↓ PostgreSQL/PostGIS + object storage
          ↓ AI/ML risk engine
          ↓ Risk + exposure + response-priority engine
          ↓ React GIS dashboard + mobile field app
          ↓ WebSocket/SMS/push/email alerts

# 4. Data Sources – Where Everything Comes From

We do not need to own a satellite or install thousands of sensors. The prototype can combine public Earth-observation datasets, weather products and our own field reports.

| Data | Recommended source | Purpose | Role | Priority |
| --- | --- | --- | --- | --- |
| Optical satellite | Copernicus Sentinel-2 / Google Earth Engine | Vegetation, soil, land-cover/change | Susceptibility/change | High |
| SAR satellite | Copernicus Sentinel-1 | Surface change; works day/night and through clouds | Change monitoring | High |
| Indian EO | ISRO/NRSC Bhuvan | Indian satellite/thematic/DEM resources | Regional mapping | High |
| Terrain | SRTM / Bhuvan DEM | Elevation, slope, aspect, curvature | Static susceptibility | Very high |
| Rainfall | NASA GPM IMERG + official feeds where available | Recent/accumulated precipitation | Trigger | Very high |
| Historical events | NASA COOLR/GLC + regional inventories | Positive labels/validation | Training | Very high |
| Soil moisture | Satellite/model products + optional sensors | Wetness before storm | Trigger | Medium |
| Roads/villages | OpenStreetMap + authoritative layers | Exposure/impact | Response | High |
| Field reports | Our own app | GPS, photos, videos, cracks, blockages | Ground validation | Very high |

# 5. Satellite Data – Exactly How We Get It

## 5.1 Sentinel-2 Optical Imagery

Sentinel-2 is multispectral optical imagery. Its MSI has 13 bands, with 10 m, 20 m and 60 m spatial resolutions. It is useful for vegetation, soil and land-cover/change features.

Define a pilot study area in NER.

Query Sentinel-2 scenes for the area/date range using Copernicus Data Space or Google Earth Engine.

Filter high-cloud scenes and apply cloud masking.

Use surface-reflectance products where possible.

Calculate features such as NDVI and selected spectral bands.

Aggregate the features to the same grid used by the ML model.

Store derived features instead of repeatedly downloading the archive.

## 5.2 Sentinel-1 SAR Imagery

Sentinel-1 uses C-band Synthetic Aperture Radar. SAR can observe day/night and is much less affected by cloud cover, which is valuable during monsoon periods. Use it for temporal surface-change/anomaly features. For the first prototype, use processed GRD products; advanced interferometry can be a later enhancement.

## 5.3 ISRO/NRSC Bhuvan

Bhuvan is ISRO/NRSC's geospatial platform. NRSC states that satellite data are hosted for visualization and that selected Resourcesat products, including AWiFS and LISS-III, are available for free download with stated time lags. Bhuvan also provides Indian thematic and DEM resources. Always verify product latency, licensing and access conditions before operational use.

## 5.4 Landsat

Landsat provides a long historical Earth-observation record. Use it mainly for long-term land-surface/change context and historical feature engineering rather than claiming it is a real-time early-warning feed.

# 6. Rainfall Data – How We Collect It

NASA GPM IMERG combines precipitation information from multiple satellites. Current documentation describes IMERG V07B coverage back to 1998 and near-real-time Early Run products with low latency. Relevant products are available at about 0.1° (~10 km) and half-hourly temporal resolution.

Rainfall in the last 30 minutes / 3 hours / 6 hours / 24 hours.

3-day and 7-day accumulated rainfall.

Rainfall intensity.

Rainfall anomaly against historical conditions.

Antecedent rainfall index – how wet the area has been before the storm.

For an operational system, combine satellite precipitation with available official/local weather observations and retain source, timestamp and latency metadata.

# 7. Terrain and Slope Data

A Digital Elevation Model (DEM) provides elevation per pixel. From it we calculate slope, aspect, curvature, relief and related terrain variables.

SRTM 1 Arc-Second Global (~30 m) via USGS/Earth Engine.

Suitable ISRO/NRSC Bhuvan DEM products.

Derived features: elevation, slope, aspect, curvature, local relief, distance to drainage and optional topographic wetness.

# 8. Historical Landslide Data – Training Labels

NASA's Cooperative Open Online Landslide Repository (COOLR) contains report-based and event-based landslide information, including the Global Landslide Catalog and Landslide Reporter Catalog. NASA also exposes a public Global Landslide Catalog export.

For each event, keep date, latitude/longitude or polygon, source, trigger when known, impacts and confidence/quality metadata.

| event_id | date | lat | lon | rain_24h | rain_7d | slope | elevation | landcover | label |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS001 | 2024-07-14 | 26.7 | 92.1 | 118 | 356 | 34° | 1320 | forest | 1 |

Label 1 = landslide observed in the defined spatial/temporal window. Negative examples must be sampled carefully to avoid data leakage and reporting bias.

# 9. Field/Citizen Reporting

GPS location and timestamp.

Photo/video upload.

Crack/slope-movement report.

Road blockage report.

Description and severity.

Citizen or field-officer role.

Offline local storage and automatic synchronization when connectivity returns.

Field reports become a validation signal. Example: AI predicts High risk and an officer then reports fresh cracks at that location; the system can increase incident priority and notify authorities.

# 10. Data Preprocessing Pipeline

Choose an initial geographic grid, e.g. 1 km × 1 km.

Reproject and spatially align all layers.

Clip datasets to the selected NER pilot areas.

Clean missing/invalid values.

Cloud-mask optical imagery.

Prepare Sentinel-1 change features.

Derive terrain variables from DEM.

Create rolling rainfall windows.

Join historical landslides to grid cells/time windows.

Build the final ML feature table.

# 11. AI/ML Model

Start with a strong, explainable tabular model rather than immediately using deep learning.

Baseline: Random Forest or XGBoost.

Inputs: slope, elevation, rainfall windows, rainfall anomaly, land cover, vegetation/change, soil moisture, historical susceptibility and satellite-change features.

Output: probability of landslide risk.

Advanced phase: add time-series/lagged features or a temporal model if the dataset supports it.

Use Sentinel-1/2 change as supporting evidence, not the sole predictor.

Example output: Risk probability = 0.86; Risk level = CRITICAL; Confidence = calibrated model confidence; Reasons = high 24h rainfall + steep slope + wet antecedent conditions.

# 12. Explainable AI

Show why a zone is high-risk. SHAP or feature-importance methods can identify the strongest contributors.

24-hour rainfall: strong contribution.

7-day rainfall: strong contribution.

Slope: strong contribution.

Previous landslide density: moderate contribution.

Satellite/vegetation change: moderate contribution.

# 13. Risk & Emergency Priority Engine

AI hazard probability alone is not enough. A high-risk empty hillside and a high-risk slope above a major road should not have the same response priority.

Conceptual priority = Hazard risk × Exposure × Vulnerability × Impact/connectivity factor

Check nearby villages, roads, bridges and critical infrastructure.

Estimate affected population/asset importance where authoritative data are available.

Prioritize locations where a landslide could cut critical connectivity.

# 14. GIS Dashboard

Live risk heatmap.

Rainfall layer.

Satellite observation/change layer.

Slope/elevation layer.

Historical landslide events.

Roads, bridges, villages and critical infrastructure.

Citizen/field reports.

Active alerts.

Clicking a zone should show probability, risk level, rainfall history, terrain, recent satellite observation, nearby historical events, field reports, affected assets and response priority.

# 15. Alerts

Real-time web dashboard alert.

Mobile push notification.

SMS for critical warnings.

Email for administrative notifications.

Optional multilingual voice/IVR as a future enhancement.

Each alert should contain location, risk level, time, reasons, affected assets, recommended action and a GIS link.

# 16. Offline/Low-Network Operation

The field application stores reports locally when there is no network, then synchronizes them when connectivity returns. This directly supports remote-area operation.

# 17. Recommended Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Frontend | React + Bootstrap + Mapbox/Leaflet | GIS command dashboard |
| Backend | Node.js + Express.js | APIs, auth, workflows |
| AI service | Python + FastAPI | Inference/data science |
| ML | XGBoost/Random Forest + SHAP | Risk prediction + explanation |
| Geospatial | Rasterio + GeoPandas + GDAL | Raster/vector processing |
| Database | PostgreSQL + PostGIS | Geospatial data |
| Storage | S3-compatible object storage | Photos/videos/raster files |
| Real-time | Socket.IO/WebSocket | Live risk/alerts |
| Deployment | Docker + cloud VM/managed services | Reproducible deployment |

# 18. How We Actually Build the Training Dataset

Collect historical landslide events from COOLR/GLC and other permitted inventories.

For each event, record location and event date.

Retrieve rainfall accumulated before the event (e.g. 3h, 24h, 72h and 7d).

Extract DEM-derived terrain features.

Extract satellite/land-cover/vegetation features from imagery around the event.

Add soil-moisture features where available.

Add distance/proximity to roads, streams and settlements where appropriate.

Create carefully sampled negative examples.

Split by geography and/or time to avoid leakage.

Evaluate precision, recall, F1, PR-AUC and probability calibration.

# 19. How We Prove the Model Works

Confusion matrix.

Precision, recall and F1.

PR-AUC because landslide events are relatively rare.

Calibration/reliability of probabilities.

Spatial holdout: test on areas not used for training.

Temporal holdout: train on earlier periods and test on later events.

Ablation: compare terrain-only, rainfall-only and combined models.

# 20. SIH MVP – What We Should Actually Build

Do not attempt full operational coverage of the entire NER during the hackathon. Build a convincing pilot for selected districts/road corridors and demonstrate how the architecture scales.

Select 1–3 representative NER study areas.

Build historical dataset: landslides + rainfall + DEM + satellite-derived features.

Train and evaluate XGBoost/Random Forest.

Build GIS risk heatmap.

Integrate rainfall data pipeline.

Add historical landslide layer.

Add GPS/photo field-reporting page.

Implement real-time alerts.

Show explainable risk reasons.

Demonstrate offline report capture and synchronization.

# 21. Example End-to-End Demo

A heavy-rain event begins in the selected pilot district.

Rainfall data update arrives.

System calculates 3h/24h/7d rainfall features.

GIS shows high rainfall over steep terrain.

AI calculates high landslide probability.

Risk engine checks nearby roads, villages and infrastructure.

Zone becomes CRITICAL because hazard and exposure are high.

Dashboard generates an alert.

Field officer receives the alert and uploads a geo-tagged crack photo.

Report is synchronized and linked to the predicted zone.

Command center shows timeline, risk reasons and affected assets.

# 22. Limitations – Be Honest in the Presentation

Satellite imagery is not continuous CCTV; optical imagery can be cloud-obscured and satellite revisit times vary.

Rainfall satellite products have finite spatial resolution and latency.

Historical landslide inventories are incomplete because many events are never reported.

The model is decision support/early warning, not a guarantee that a landslide will or will not happen.

Ground validation and domain-expert review are essential before operational deployment.

# 23. How This Becomes a Big Product

Scale from pilot districts to all NER states.

Add more official weather/sensor feeds.

Add road-closure and traffic information.

Add flash-flood and hydrology modules.

Add satellite change detection.

Add AI-assisted field-photo analysis.

Add multilingual warnings.

Provide APIs for government systems.

Create district-level command centers and historical risk analytics.

# 24. Suggested Team Division

| Role | Main work | Output |
| --- | --- | --- |
| AI/ML | Dataset, features, model, evaluation | Risk model |
| GIS/Data | Satellite/DEM/rainfall ETL | Geospatial pipeline |
| Backend | APIs, DB, alerts | Node/Express + PostGIS |
| Frontend | Maps and dashboard | React command center |
| Mobile/Field | Reports + offline sync | Field app/PWA |
| DevOps | Docker/deployment/monitoring | Cloud demo |

# 25. Final Recommended Architecture

CCTV-like live monitoring is NOT the model here. The system is a geospatial early-warning platform:

Satellite + DEM + Rainfall + Soil moisture + Historical landslides + Field reports
                         ↓
                 Data preprocessing
                         ↓
                 Feature generation
                         ↓
              AI/ML hazard prediction
                         ↓
              Exposure/priority engine
                         ↓
                  GIS heatmap
                         ↓
             Alerts + field response
                         ↓
               Feedback / validation

# 26. Key Sources

SIH26001 problem statement archive: https://github.com/suhaibafther/SIH-2026-PROBLEM-ARCHIVE

SIH 2026 problem statements dataset: https://github.com/jeevansai-hub/SIH-2026-

ISRO/NRSC Bhuvan: https://bhuvan.nrsc.gov.in/

NRSC Satellite Data Services: https://www.nrsc.gov.in/nrscnew/Services_Bhuvan_SatelliteDataServices.php?lang_code=en

Bhuvan Store: https://bhuvan-app1.nrsc.gov.in/2dresources/bhuvanstore.php

Copernicus Data Space: https://dataspace.copernicus.eu/

Sentinel-1: https://dataspace.copernicus.eu/data-collections/copernicus-sentinel-missions/sentinel-1

Google Earth Engine Data Catalog: https://developers.google.com/earth-engine/datasets/catalog/

USGS Landsat Data Access: https://www.usgs.gov/landsat-missions/landsat-data-access

USGS EarthExplorer: https://earthexplorer.usgs.gov/

NASA GPM IMERG: https://gpm.nasa.gov/data/imerg

NASA GPM Data Directory: https://gpm.nasa.gov/data/directory

NASA COOLR: https://gpm.nasa.gov/applications/landslides/coolr

NASA Landslide Reporter: https://science.nasa.gov/citizen-science/landslide-reporter/

NASA Global Landslide Catalog Export: https://data.nasa.gov/dataset/global-landslide-catalog-export

# 27. One-Sentence Project Pitch

“We combine terrain, rainfall, satellite observations, historical landslides and ground reports to continuously estimate where landslide risk is increasing, explain why, warn the right people and prioritize emergency response.”